PURGENVENT Model 7900AAV

- 2.1	AIR VENTING VALVES

    - Series:  PURGEnVENT as manufactured by AGF Manufacturing, Inc.

          - Product:  M7900AAV Automatically Vent Air from the System. Automatic Air Venting Valve is a 1 inch (25 mm) ball valve, strainer, air vent, and purge assembly.

            - Standard:  UL's "Product iQ" listing or "Approval Guide," published by FM Global, listing.  NFPA 13, 13R, 13D (as applicable).

            - Pressure Rating:  300 psig (2070 kPa), 175 psi (1205 kPa) for air vent.

            - Body Material:  Forged brass body.

            - Components:  Integrated ball valve, stainless steel strainer, purge valve with hose connection (Field adjustable), thread cap with lanyard, automatic air vent.

            - Inlet Size:  1 inch NPT.

            - Outlet Size:  3/4 inch (19 mm) NPT.