PURGENVENT Model 7950ILV

- 2.1	SPRINKLER SPECIALTY PIPE FITTINGS

        - Main Line Air Vent:

          - Product:  M7950ILV In-Line Automatic Air Venting Valve. The separation chamber and air scoop is designed to force a separation of air from the water as the system fills.

            - Standard:  UL's "Fire Protection Equipment Directory" listing or "Approval Guide," published by FM Global, listing.  NFPA 13, 13R, 13D (as applicable).

            - Includes M7900AAV.

            - Pressure Rating:  300 psig (2070 kPa).

            - Separation Chamber:  Carbon steel body with threaded fitting for machined brass Air Scoop attachment. Powder coated Safety Red.

** NOTE TO SPECIFIER **  Delete inlet and outlet size not required.

            - Inlet and Outlet Size:

              - 2 inches (51 mm) Grooved ends.

              - 2.5 inches (64 mm) Grooved ends.

              - 3 inches (76 mm) Grooved ends.

              - 4 inches (102 mm) Grooved ends.

              - 6 inches (152 mm) Grooved ends.

              - 8 inches (203 mm) Grooved ends.