PURGENVENT Model 7930ECA

- 2.1	SPRINKLER SPECIALTY PIPE FITTINGS

        - Main Line Air Vent:

          - Product:  M7930ECA Automatic Air Venting Valve with End Cap. The Model 7930ECA includes an end cap so it is easily installed onto the end of branch lines with a coupling.

            - Standard:  UL's "Fire Protection Equipment Directory" listing or "Approval Guide," published by FM Global, listing.  NFPA 13, 13R, 13D (as applicable).

            - Includes M7900AAV.

            - Pressure Rating:  300 psig (2070 kPa).

            - End Cap:  Painted, Ductile Iron with Groove end and 1” NPT offset connection.

** NOTE TO SPECIFIER ** Delete inlet size not required.

            - Inlet Size:

              - 2 inches (51 mm) Grooved end.

              - 2.5 inches (64 mm) Grooved end.

              - 3 inches (76 mm) Grooved end.

              - 4 inches (102 mm) Grooved end.

              - 6 inches (152 mm) Grooved end.

              - 8 inches (203 mm) Grooved end.